Chapter 7

# Timers

Correct system timing is a fundamental requirement for the proper operation of a real-time application. The timing definition can dictate how the data information processed during the execution of the application program. The clock implementations vary between devices in the MSP430 family. Each device provides different clock sources, controls and uses. This chapter discusses the clock controls included in the platforms used.

The MSP430 4xx family has two general-purpose 16-bit or 8-bit counters and event timers, named Timer_A, Timer_B, and a Basic Timer. The Basic Timer module is only implemented in ‘4xx devices. The 2xx device family also has Timer_A and Timer_B, but the clock signals are provided by the basic clock module+.

The timers may receive an internal or external clock. Timer_A and Timer_B also include multiple independent capture and compare blocks, with interrupt capabilities.

The capture and compare blocks are suited for applications such as timed events and Pulse Width Modulator (PWM) respectively.

Topic	Page

7.1 Timers Introduction 7-3 7.2 Basic Timer 1 7-4 7.2.1 Registers 7-5 7.3 Timer_A and Timer_B 7-6 7.3.1 Operating modes characteristics 7-7 7.3.2 Timer reset 7-8 7.3.3 Timer_A registers 7-8 7.4 Capture/Compare blocks 7-9 7.4.1 Capture mode 7-9 7.4.2 Compare mode 7-10 7.4.3 Capture/Compare blocks registers 7-12 7.5 Timer_A Interrupts 7-13 7.6 Timer_B special features 7-13 7.6.1 Timer_B registers 7-14 7.7 Laboratory 3: Timers use 7-16 7.7.1 Lab3A_1: Memory clock with Basic Timer1 7-16 7.7.2 Lab3A_2: Real Time Clock with Basic Timer1 7-20 7.7.3 Lab3B: Memory Clock with Timer_A 7-23 7.7.4 Lab3C: Buzzer tone generator 7-26 7.7.5 Lab3D: Frequency measurement 7-32 7.8 Quiz 7-38 7.9 FAQs 7-39

## Timers Introduction

In Chapter 5 – Device Systems and Operating Modes described the different system clocks sources that allow the CPU and peripherals to operate, depending on the device in the MSP430 family. This chapter will focus mainly on the configuration and operation of timers and their different uses.

This chapter describes the basic clock module+ (BCS+) three byte-addressable registers, all of which are fully software controllable. These allow the clock sources to be configured by one or two oscillators (depending on the device) or by external crystals or resonators, as wells as by the internal digitally controlled oscillator (DCO). The DCO allows a working frequency up to 16 MHz, lower power consumption and lower internal oscillator start up time.

The system timing is fundamental to almost any embedded application. The timers are used in applications to:

    - Generate fixed-period time events;

    - Allow a periodic wakeup from sleep mode;

    - Count transitional signal edges;

    - Replace delay loops with timer calls to allow the CPU to sleep, consuming much less power.

The MSP430 devices contain several timer modules, each one suitable for different types of tasks. Chapter 5 described the timer modules as Basic Clock Module+ implemented in the MSP430x2xx family and the Watchdog Timer present in all families of MSP430 devices. This chapter will focus on:

    - Basic Timer 1 module (implemented only in the MSP430x4xx family);

    - Timer_A;

    - Timer_B.

The clock signals of the MSP430x4xx family devices are controlled by two sets of registers. The first set of registers configures the low-frequency signals for use by peripheral modules. These registers, namely Basic Timer Control Register (BTCTL), Basic Timer Counter 1 (BTCNT1) and Basic Timer Counter 1(BTCNT2) control the Basic Timer 1 module.

The second set of registers is dedicated to the configuration of the general-purpose clocks system. They comprise of the System Clock Control (SCFQCTL), System Clock Frequency Integrator 0 (SCFI0), System Clock Frequency Integrator 1 (SCFI1), and the two Frequency Locked Loop control registers, FLL+CTL0 and FLL+CTL1.

## Basic Timer 1

The Basic Timer 1 module is formed by two independent 8-bit timers: Basic Timer 1 Counter 1 (BTCNT1) and Basic Timer 1 Counter 2 (BTCNT2), which can be used in cascade to form a 16-bit timer (software selectable by BTCTL register configuration).

The main characteristics of this module are:

    - Clock for LCD module;

    - Suitable for a RTC implementation;

    - Basic interval timer;

    - Simple interrupt capability.

These timers/counters have different applications:

    - BTCNT1: Used to generate the frame frequency for the LCD controller:

      - Read/write 8-bit register;

      - Clock source: ACLK;

      - Frame frequency selectable by software (BTFRFQx bits on the BTCTL register) based on the ACLK division (x);

      - fLCD = ACLK/x.

    - BTCNT2: Used as a programmable frequency divider with interrupt capability, to provide periodic CPU interrupts and/or a Real Time Clock system.

      - Read/write 8-bit register;

      - Three possible clock sources: ACLK or SMCLK, or ACLK/256 when cascaded with BTCNT1 (selected by the BTSSEL and BTDIV bits on the BTCTL register);

      - Sources the Basic Timer1 interrupt BTIFG, with an interval  selected by the BTIPx bits in the BTCTL register;

Figure 7-1. Basic Timer 1 block diagram.

### Registers

BTCTL, Basic Timer 1 Control Register

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BTSSEL | BTHOLD | BTDIV | BTFRFQ1 | BTFRFQ0 | BTIP2 | BTIP1 | BTIP0 |

| Bit |  | Description |
| --- | --- | --- |
| 7 | BTSSEL | BTCNT2 clock select (together with the BTDIV bit) |
| 6 | BTHOLD | Basic Timer 1 hold: BTHOLD = 0  BTCNT1 and BTCNT2 active BTHOLD = 1  BTCNT1 hold, if BTDIV = 1 BTHOLD = 1  BTCNT1 and BTCNT2 hold |
| 5 | BTDIV | Basic Timer 1 clock divider (together with the BTSSEL bit selects the BTCNT2 clock signal): BTSSEL BTDIV = 0 0  ACLK BTSSEL BTDIV = 0 1  ACLK/256 BTSSEL BTDIV = 1 0  SMCLK BTSSEL BTDIV = 1 1  ACLK/256 |
| 4-3 | BTFRFQx | LCD frame frequency: BTFRFQ1 BTFRFQ0 = 0 0  fACLK/32 BTFRFQ1 BTFRFQ0 = 0 1  fACLK/64 BTFRFQ1 BTFRFQ0 = 1 0  fACLK/128 BTFRFQ1 BTFRFQ0 = 1 1  fACLK/256 |
| 2-0 | BTIPX | Basic Timer 1 interrupt interval: BTIP2 BTIP1 BTIP0 = 0 0 0  fCLK2 / 2 BTIP2 BTIP1 BTIP0 = 0 0 1  fCLK2 / 4 BTIP2 BTIP1 BTIP0 = 0 1 0  fCLK2 / 8 BTIP2 BTIP1 BTIP0 = 0 1 1  fCLK2 / 16 BTIP2 BTIP1 BTIP0 = 1 0 0  fCLK2 / 32 BTIP2 BTIP1 BTIP0 = 1 0 1  fCLK2 / 64 BTIP2 BTIP1 BTIP0 = 1 1 0  fCLK2 / 128 BTIP2 BTIP1 BTIP0 = 1 1 1  fCLK2 / 256 |

IE2, Interrupt Enable Register 2

| 7 |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BTIE |  |  |  |  |  |  |  |

| Bit |  | Description |
| --- | --- | --- |
| 7 | BTIE | Basic Timer 1 interrupt enable when BTIE = 1 |

IFG2, Interrupt Flag Register 2

| 7 |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BTIFG |  |  |  |  |  |  |  |

| Bit |  | Description |
| --- | --- | --- |
| 7 | BTIFG | Basic Timer 1 interrupt flag BTIFG = 1 when interrupt pending |

## Timer_A and Timer_B

Timer_A and Timer_B are two general-purpose 16-bit counters and event timers. There are slight differences between the two timers that will be explained in this section.

Both timers feature:

    - Asynchronous 16-bit timer/counter with four operating modes:

      - Timer_A length: 16 bits;

      - Timer_B length: programmable to be 8, 10, 12, or 16 bits.

      - The timer/counter register, TAR (Timer_A) or TBR (Timer_B) -from now on described as TxR- increments or decrements (depending on mode of operation) with each rising edge of the clock signal;

      - The timer can generate an interrupt when it overflows;

      - Wide interrupt interval range: 1/MCLK to 32 seconds.

    - Selectable and configurable clock source:

      - ACLK, SMCLK, or externally via TxCLK or INCLK (selected with the TASSELx bits);

      - The selected clock source may additionally be divided by 2, 4, or 8 (IDx bits configuration).

    - Configurable capture/compare registers:

      - Timer_A has three or five capture/compare registers;

      - Timer_B has three or seven capture/compare registers;

      - Timer_B capture/compare registers can be grouped.

    - Configurable outputs and several internal connections to other modules, allowing a faster response because no cycles are wasted while the ISR loads/executes and avoids CPU wakeup, which in turn saves power:

      - Outputs with Pulse Width Modulation (PWM) capability;

      - Comparator_A;

      - Direct Memory Access;

      - Digital-to-Analogue Converter (DAC12);

    - Asynchronous input and output latching:

      - Timer_A capture/compare registers are not buffered, being updated immediately when written to;

      - Timer_B capture/compare registers are double-buffered with synchronized loading.

    - Interrupt vector register for fast decoding of all Timer_A and Timer_B interrupts.

      - TACCR0 (or TBCCR0) interrupt vector for TACCR0 (or TBCCCR0) CCIFG;

      - TAIV (or TBIV) interrupt vector for the remainder CCIFG flags and TAIFG (or TBIFG).

The timer has four modes of operation as described in Table 7−1, selected with the MCx bits in the Timer_A or Timer_B Control Registers.

Table 7-1. Timer_A and Timer_B operating modes.

| MCx | Mode | Description |
| --- | --- | --- |
| 0 0 | Stop | The timer is halted |
| 0 1 | Up | Up counting repeatedly mode (from 0x0000 to the value in the TACCR0 or TBCCR0 register) |
| 1 0 | Continuous | Continuous counting repeatedly mode (from 0x0000 to 0xFFFF) |
| 1 1 | Up/down | Up/down counting repeatedly mode (from 0x0000 to the value in the TACCR0 or TBCCR0 register and back down to zero) |

Because the main characteristics provided by the two timers are similar, the following sections will only discuss the features of Timer_A. To use these features with Timer_B, it is only necessary to configure the appropriate registers. The register descriptions for Timer_A are valid for Timer_B.

### Operating modes characteristics

Up mode

The main characteristics of up mode are:

    - TAR counts up with each clock pulse until it reaches the value in the TACCR0 (or TBCCR0) register;

    - The TACCR0 (or TBCCR0) interrupt flag, CCIFG, is set when the timer counts to the TACCR0 (or TBCCR0) value;

    - When it reaches this value, EQU0 = 1 (restarts TAR counting from zero);

    - The TAIFG (or TBIFG) interrupt flag is set when the timer counts from TACCR0 (or TBCCR0) value to zero.

      - Interrupt period: tINT = 1/[fCLK/Prescaler/(TxCCR0+1)];

        - tINT: TxIFG interrupt period [sec];

        - fCLK: Clock source frequency [Hz];

        - Prescaler: Divider (IDx bits).

Continuous mode

The main characteristics of continuous mode are:

    - TxR counts up with each clock pulse till 0xFFFF (65536 counts);

    - When it reaches this value, at the next clock pulse it will restart the TxR counting from zero;

    - The TxIFG (or TxIFG) interrupt flag is set when the timer counts from 0xFFFF to zero.

      - Interrupt period: tINT = 1/[fCLK/Prescaler/65536];

    - (Correct only for TAR; for TBR 4 there are different end values. See User’s Guide for additional details).

Up/down mode

The main characteristics of the up/down mode are:

    - TxR counts up with each clock pulse until it reaches the value in the TxCCR0 register;

    - The TxCCR0 interrupt flag, CCIFG, is set when the timer counts from TxCCR0 − 1 to TxCCR0;

    - When it reaches this value, the counting is inverted, starting at the next clock pulse to decrement till zero;

    - The interrupt flag TxIFG is set when the timer completes counting down from 0x0001 to 0x0000.

      - Interrupt period: tINT = 1/[fCLK/Prescaler/(TxCCR02];

### Timer reset

The timer can be reset using the following operations:

    - Writing 0 at the TAR (or TBR) register;

    - Writing 0 at the TACCR0 (or TBCCR0) register provided that the timer is not in continuous mode;

    - Setting the TACLR (or TBCLR) bit in the Timer Control Register (TACTL or TBCTL).

### Timer_A registers

The description of the following registers relates to Timer_A registers. Timer_B registers have the same features, but the Control Register has different bit names and provides additional features. For more details, refer to MSP430x4xx User’s Guide <slau056g.pdf> Chapter 16 – Timer_B to find the differences. The special features of Timer_B are described in section 7.6.

TACTL, Timer_A Control Register

| 15 |  |  |  |  | 10 | 9 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Unused |  |  |  |  |  | TASSEL1 | TASSEL0 |

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ID1 | ID0 | MC1 | MC0 | Unused | TACLR | TAIE | TAIFG |

| Bit |  | Description |
| --- | --- | --- |
| 9-8 | TASSELx | Timer_A clock source: TASSEL1 TASSEL0 = 00  TACLK TASSEL1 TASSEL0 = 01  ACLK TASSEL1 TASSEL0 = 10  SMCLK TASSEL1 TASSEL0 = 11  INCLK |
| 7-6 | IDx | Clock signal divider: ID1 ID0 = 00  / 1 ID1 ID0 = 01  / 2 ID1 ID0 = 10  / 4 ID1 ID0 = 11  / 8 |
| 5-4 | MCx | Clock timer operating mode: MC1 MC0 = 00  Stop mode MC1 MC0 = 01  Up mode MC1 MC0 = 10  Continuous mode MC1 MC0 = 11  Up/down mode |
| 2 | TACLR | Timer_A clear when TACLR = 1 |
| 1 | TAIE | Timer_A interrupt enable when TAIE = 1 |
| 0 | TAIFG | Timer_A interrupt pending when TAIFG = 1 |

## Capture/Compare blocks

Both TIMER_A (and TIMER_B) contain independent capture and compare blocks, TACCRX (or TBCCRX) that may be used to capture the timer register contents, as they are at time of an event, or to generate an event when the timer register contents correspond to capture/compare register contents, e.g. to generate time intervals.

The mode is selected by the mode bit CAP in their individual Capture/Compare Control register, TACCTLx (or TBCCTLx).

### Capture mode

The capture mode is used to measure the period of timed events, with minimal CPU intervention.

Capture mode configuration is achieved by carrying out the following steps:

    - Set CAP bit to select the capture mode;

    - Set SCS bit to synchronize the capture with the next timer clock (recommended to avoid race conditions);

    - The input signal is sampled by the CCIxA (or CCIxB) input, selected by the CCISx bits in the Capture/Compare Control Register, TACCTLx (or TBCCTLx);

    - The capture edge of the input signal (rising, falling, or both) is selected by the CMx bits;

    - When the appropriate edge is detected on the selected input line, the value in the Timer register is latched into the TACCRx (or TBCCRx) register, providing a time mark for the event;

    - The interrupt flag CCIFG is set;

    - The bit COV (=1) controls an overflow event when a second capture is performed before the value from the first capture is read.

### Compare mode

The compare mode is used for pulse generation or interrupts at specific time intervals. One of its common applications is to generate Pulse Width Modulation (PWM) output signals.

Compare mode operation is configured as follows:

    - Reset CAP bit to select compare mode;

    - TxR counts to the value programmed in the TxCCRx register;

    - When the timer value is equal to the value in the TxCCRx register, an interrupt is generated:

      - Interrupt flag CCIFG is set;

      - Internal signal EQUx = 1 (x is the number of the CCR channel).

    - EQUx affects the output compare signal OUTx according to the output mode (defined by the OUTMODx bits in the TxCCTLx register – see Table 7-2).

    - The input signal CCI is latched into SCCI.

The output unit is used to generate output signals (such as PWM signals) based on the EQU0 and EQUx signals. There are eight different operating modes numbered from 0 to 7. The OUTx signal changes with the rising edge of the timer clock for all modes, except mode 0.

The output operating modes are described in Table 7−2 and the examples of the output signals for different timer modes are presented in Figure 7-2.

Output operating modes uses:

    - Modes 2, 3, 6 and 7: generation of PWM output signals:

      - Mode 3: active PWM signal at low state;

      - Mode 7: active PWM signal at high state;

      - Modes 2 and 6: complementary PWM signals separated by a deadband when both signals are reset (application example:  H-bridges for bi-directional motor drive).

    - Modes 1 and 5: single event generation;

    - Mode 4: signal with half the frequency of the timer signal.

Table 7-2. Output operating modes.

| OUTMODx | Mode | Description |
| --- | --- | --- |
| 000 | Output | The output signal OUTx is defined by the bit OUTx |
| 001 | Set | OUTx = 1  timer = TACCRx OUTx = 0  timer = 0 or until another output mode is selected and affects the output |
| 010 | Toggle/Reset | OUTx = toggle  timer = TACCRx OUTx = 0  timer = TACCR0 |
| 011 | Set/Reset | OUTx = 1  timer = TACCRx OUTx = 0  timer = TACCR0 |
| 100 | Toggle | OUTx = toggle  timer = TACCRx The output period is double the timer period |
| 101 | Reset | OUTx = 0  timer = TACCRx OUTx = 1  another output mode is selected and affects the output |
| 110 | Toggle/Set | OUTx = toggle  timer = TACCRx OUTx = 1  timer = TACCR0 |
| 111 | Reset/Set | OUTx = 0  timer = TACCRx OUTx = 1  timer = TACCR0 |

Figure 7-2. Output examples for different timer modes.

| Timer Mode Output mode | Up mode | Continuous mode | Up/Down mode |
| --- | --- | --- | --- |

1: Set

2: Toggle/Reset

3: Set/Reset

4: Toggle

5: Reset

6: Toggle/Set

7: Reset/Set

### Capture/Compare blocks registers

The description of the following registers relates to Timer_A registers. Timer_B registers have the same features, but the Capture/Compare Control Register have different bit names and provide additional features. Refer to MSP430x4xx User’s Guide <slau056g.pdf> Chapter 16 – Timer_B to find the differences. The special features of Timer_B are described in section 7.6.

TACCTLx, Timer_A Capture/Compare Control Register

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CM1 | CM0 | CCIS1 | CCIS0 | SCS | SCCI | Unused | CAP |

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| OUTMOD2 | OUTMOD1 | OUTMOD0 | CCIE | CCI | OUT | COV | CCIFG |

| Bit |  | Description |
| --- | --- | --- |
| 15-14 | CMx | Capture mode: CM1 CM0 = 00  No capture CM1 CM0 = 01  Capture on rising edge CM1 CM0 = 10  Capture on falling edge CM1 CM0 = 11  Capture on both edges |
| 13-12 | CCISx | Capture/compare input select: CCIS1 CCIS0 = 00  CCIxA CCIS1 CCIS0 = 01  CCIxB CCIS1 CCIS0 = 10  GND CCIS1 CCIS0 = 11  Vcc |
| 11 | SCS | Synchronize capture input signal with timer clock: SCS = 0  Asynchronous capture SCS = 1  Synchronous capture |
| 10 | SCCI | Synchronized capture/compare input |
| 8 | CAP | Mode: Capture mode  CAP = 1 Compare mode  CAP = 0 |
| 7-5 | OUTMODx | Output mode: OUTMOD2 OUTMOD1 OUTMOD0 = 000  Bit OUT OUTMOD2 OUTMOD1 OUTMOD0 = 001  Set OUTMOD2 OUTMOD1 OUTMOD0 = 010  Toggle/Reset OUTMOD2 OUTMOD1 OUTMOD0 = 011  Set / Reset OUTMOD2 OUTMOD1 OUTMOD0 = 100  Toggle OUTMOD2 OUTMOD1 OUTMOD0 = 101  Reset OUTMOD2 OUTMOD1 OUTMOD0 = 110  Toggle / Set OUTMOD2 OUTMOD1 OUTMOD0 = 111  Reset / Set |
| 4 | CCIE | Capture/compare interrupt enable when CCIE = 1. |
| 3 | CCI | Capture/compare input |
| 2 | OUT | Output state |
| 1 | COV | Capture overflow when COV = 1 |
| 0 | CCIFG | Capture/compare interrupt flag CCIFG = 1 when interrupt pending |

## Timer_A Interrupts

Interrupt characteristics:

    - Capture mode:

      - Any CCIFG flag is set when a timer value is captured in the associated TACCRx register.

    - Compare mode:

      - Any CCIFG flag is set if TAR counts to the associated TACCRx value.

    - Software may also set or clear any CCIFG flag;

    - All CCIFG flags request an interrupt when their corresponding CCIE bit and the GIE bit are set.

There are two interrupt vectors associated with Timer_A:

    - TACCR0 interrupt vector for TACCR0 CCIFG:

      - TACCR0 CCIFG flag has the highest Timer_A interrupt priority;

      - The TACCR0 CCIFG flag is automatically reset when the TACCR0 interrupt request is serviced.

    - TAIV interrupt vector for TACCR1 CCIFG to TACCR4 CCIFG and TAIFG:

      - The flags are prioritized and combined to source a single interrupt vector (decreasing priority);

      - The interrupt vector register TAIV is used to determine which flag requested an interrupt;

      - Disabling Timer_A interrupts does not affect the TAIV value;

      - Any access, read or write, of the TAIV register automatically resets the highest pending interrupt flag;

      - If another interrupt flag is set, another interrupt is immediately generated after servicing the initial interrupt.

## Timer_B special features

Timer_B presents some special features:

    - Programmable length of the TBR register (equivalent to TAR in Timer_A) to be 8, 10, 12, or 16 bits:

      - Configurable through CNTLx bits selection in TBCTL (equivalent to TACTL in Timer_A);

      - The maximum count value, TBR(max), for the selectable lengths is 0FFh, 03FFh, 0FFFh, and 0FFFFh, respectively;

    - Three or seven capture/compare blocks TBCCRx;

    - Double-buffered compare latches with synchronized loading:

      - In Timer_A, the signal generation in compare mode may cause noise during compare period updates, because the TACRRx value is used directly to compare with timer value;

      - To avoid this condition, the compare latches, TBCLx, buffered by TBCCRx, hold the data for the comparison to the timer value in compare mode;

      - The CLLDx bits in the TBCCTLx register configure the timing of the transfer from TBCCRx to TBCLx.

    - Grouping channels capability:

      - Multiple compare latches may be grouped together for simultaneous updates using the TBCLGRPx bits;

      - Two conditions are required:

        - All TBCCRx registers must be updated;

        - The load event controlled by the CLLDx bits must occur.

    - All outputs can be put into a high-impedance state:

      - TBOUTH = 1 put Timer_B outputs into a high-impedance state, allowing higher security and lower delay time answering to failures.

    - The SCCI bit function is not implemented.

### Timer_B registers

This section describes only the Timer_B bits of the registers TBCTL and TBCCTL that differ from Timer_A features. Refer to Timer_A registers description to determine the common function. The different bits are highlighted in bold.

TBCTL, Timer_B Control Register

| 15 | 14 |  | 13 | 12 | 11 | 10 | 9 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unused | TBCLGRP1 | TBCLGRP0 |  | CNTL1 | CNTL0 | Unused | TBSSEL1 | TBSSEL0 |

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ID1 | ID0 | MC1 | MC0 | Unused | TBCLR | TBIE | TBIFG |

| Bit |  | Description |
| --- | --- | --- |
| 14-13 | TBCLGRPx | TBCLx group: TBCLGRP1 TBCLGRP0 = 00  Each TBCLx latch loads independently TBCLGRP1 TBCLGRP0 = 01  TBCL1+TBCL2 (update control: TBCCR1 CLLDx)  TBCL3+TBCL4 (update control: TBCCR3 CLLDx)  TBCL5+TBCL6 (update control: TBCCR5 CLLDx)  TBCL0 independent TBCLGRP1 TBCLGRP0 = 10  TBCL1+TBCL2+TBCL3 (update control: TBCCR1 CLLDx)  TBCL4+TBCL5+TBCL6 (update control: TBCCR4 CLLDx)  TBCL0 independent TBCLGRP1 TBCLGRP0 = 11  TBCL0+TBCL1+TBCL2+TBCL3+TBCL4+TBCL5+TBCL6 (update control: TBCCR1 CLLDx) |
| 12-11 | CNTLx | Counter Length: CNTL1 CNTL0 = 00  16-bit, TBR(max) = 0FFFFh CNTL1 CNTL0 = 01  12-bit, TBR(max) = 0FFFh CNTL1 CNTL0 = 10  10-bit, TBR(max) = 03FFh CNTL1 CNTL0 = 11  8-bit, TBR(max) = 0FFh |

TBCCTLx, Timer_B Capture/Compare Control Register

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CM1 | CM0 | CCIS1 | CCIS0 | SCS | CLLD1 | CLLD0 | CAP |

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| OUTMOD2 | OUTMOD1 | OUTMOD0 | CCIE | CCI | OUT | COV | CCIFG |

| Bit |  | Description |
| --- | --- | --- |
| 10-9 | CLLDx | Compare latch load: CLLD1 CLLD0 = 00  TBCLx loads on write to TBCCRx CLLD1 CLLD0 = 01  TBCLx loads when TBR counts to 0 CLLD1 CLLD0 = 10  TBCLx loads when TBR counts: - to 0 (up/continuous mode); - to TBCL0 or to 0 (up/down mode) CLLD1 CLLD0 = 11  TBCLx loads when TBR counts: - to TBCLx |

## Laboratory 3: Timers use

This hands-on laboratory consists of configuring the LCD_A controller of the MSP430FG4618 device of the Experimenter’s board to put a message on the display. Like the previous exercise (Lab2), this laboratory is composed of some sub tasks. This laboratory has been developed only for the Code Composer Essentials version 3 software development tool.

### Lab3A_1: Memory clock with Basic Timer1

Project files

- C source files:	Chapter 7 > Lab3 > Lab3a_1_student.c

Chapter 7 > Lab3 > LCD_defs.h

- Solution file:	Chapter 7 > Lab3 > Lab3a_1_solution.c

Overview

This laboratory implements a memory clock using the features provided by Timer1. The clock is updated once every second by the Basic Timer1 interrupt service routine (ISR). This procedure also performs switching of LED1. In order to evaluate the execution time of the routine, LED2 is kept active during the execution of the ISR. When the ISR has completed, the device goes into low power mode, until the new interrupt wakes it up.

A. Resources

This application sets Basic Timer1 to generate an interrupt once every second. The interrupt service routine generated by this peripheral is required to update the clock stored in memory. Moreover, it must refresh the content of the clock displayed on the LCD.

Thus, the system resources used by this application are:

    - Basic Timer1;

    - I/O ports;

    - LCD;

    - Interrupts;

    - Low power modes.

The default configuration of the FLL+ is used, so, all the clock signals required for the operation of the components of the device assume their default values.

B. Software application organization

The first task is to disable the Watchdog Timer. It should be stated that this feature, when used correctly, makes the application more robust.

The resources needed for the LCD are all configured. This code is given, since its operation will be analysed in a later laboratory. Once the LCD configured, it is cleared by the execution of the routine LCD_all_off().

The memory clock consists of setting three global variables: hour, min, and sec, all of the type unsigned char, used to store the hours, minutes and seconds values elapsed respectively since the beginning of the execution of the application. These variables are initialized with zero values.

The LCD is refreshed at startup to show the initial clock value.

LED1 is used as an indicator of Basic Timer1 ISR execution. The execution time can be determined through it. In addition, LED2 state switches whenever the Basic Timer1 ISR is executed.

The Basic Timer1 is set to generate an interrupt once every second.

The routine main() ends with the interrupts global activation and puts the device in low power mode, awaiting the next interrupt.

Basic Timer1 ISR begins by activating LED2, indicating the beginning of the routine execution and then switches the state of LED1. The counters are updated in cascade and their contents updated on the LCD, through routines LCD_sec(), LCD_min() and LCD_hour(). The routine ends with switching the state of the clock separation points. Finally, LED2 is turned off.

C. System configuration

    - Disabling the Watchdog Timer

The Watchdog Timer is disabled with the objective of reducing energy consumption, but giving up the protection afforded by it. This peripheral is configured by the WDTCTL register. Its access is protected by a password. What is the value to write to disable it?

WDTCTL = _______________;

    - FLL+ configuration

A 32.768 kHz crystal is applied to the oscillator LFXT1. Since it is possible to select the internal capacitors using software, what is the value to write to the FLL_CTL0 configuration register to select the 8 pF capacitors?

FLL_CTL0 |= ____________;

Taking into consideration the change mentioned earlier to the FLL+ module, what are the frequencies of each of the clock signals?

ACLK = _________________;

MCLK = _________________;

SMCLK = ________________;

    - LED ports configuration

LED1 and LED2 are connected to ports P2.2 and P2.1 respectively. How should they be configured so that just the bits related to these ports have digital output functions?

P2DIR = _________________;

How should the P2OUT register be configured so that the application starts with LED1 on and LED2 off?

P2OUT = _________________;

    - Basic Timer1 configuration

Basic Timer1 should generate an interrupt once every second. It uses two counters in series, so that the input of the BTCNT2 counter is the output of the BTCNT1 counter divided by 256. The BTCNT1 counter input is the ACLK with a 32.768 kHz frequency. If the selected output of the BTCNT2 counter is divided by 128, what is the time period associated with the Basic Timer1 interrupt? _________

What are the values to write in the configuration registers?

BTCTL = ________________;

IE2 = __________________;

    - Low power modes

The task simply updates the counters periodically and refreshes the LCD contents. It is possible to configure the registers for an energy-efficient operation.

Which low power mode should be used? _____________

Which system clocks are activated in the low power mode selected? _____________________

D. Analysis of operation

Knowing the values and configurations of each register, complete the file LAB3a_1_student.c. The complete solution can be found in the file LAB3a_1_solution.c. One of these files should be included in the building of the project. Afterwards, compile and debug the project.

    - System clocks inspection

The MCLK, SMCLK and ACLK system clocks are available at ports P1.1, P1.4 and P1.5 respectively. These ports are located on the SW2, RESET_CC and VREG_EN lines, which are available on the H2 Header pins 2, 5 and 6. All these resources are available because the Chipcon RF module is not installed and SW2 is not used.

Using the Registers view, set bits 1, 4 and 5 of P1SEL and P1DIR registers, to choose the secondary function of these ports configured as outputs. By connecting an oscilloscope to those lines, it is possible to monitor the clock signals.

What are the values measured for each of the system clocks?

ACLK: ________________

SMCLK: _______________

MCLK: ________________

    - ISR execution time

The Basic Timer1 ISR execution time is fundamental to energy conservation, in order to extend the life of the system battery. The routine execution time can be measured by connecting the oscilloscope to port P2.1, which controls LED2. This output is available on pin 2 of Header H4.

The execution time of this routine varies with the number of the counter updates and respective updates to the LCD. What are the times measured for each of these situations and what their frequencies?

Seconds update: ______ with a time period of  ______

Seconds and minutes update: ______ with a time period of  ______

LCD fields update: ______ with a time period of  ______

If the developer chooses to update all the LCD fields at each interrupt, the time required is much greater than the solution presented. Efficient programming contributes to a reduction in the system power consumption.

    - Measurement of electrical current drawn

The power consumption was discussed in the previous point. The electrical power required by the system during operation is measured by replacing the jumper on the Header PWR1 by an ammeter, which indicates the electric current taken by device during operation.

What is the value read? __________

| MSP-EXP430FG4618 | SOLUTION |
| --- | --- |
| Implement a memory clock using the features supported by Timer1. |  |

    - Disabling the Watchdog Timer:

WDTCTL = WDTPW | WDTHOLD;     // Stop WDT

    - FLL+ configuration:

FLL_CTL0 |= XCAP18PF;         // Set load cap for 32k xtal

    - LED ports configuration:

P2DIR |= 0x06;                           // P2.2 and P2.1 as output

P2OUT |= 0x04;                // LED1 on and LED2 off

    - Basic Timer1 configuration:

BTCTL = BTDIV | BT_fCLK2_DIV128; // (ACLK/256)/128

IE2 |= BTIE;              // Enable Basic Timer1 interrupt

    - Low power modes:

BIS_SR(LPM3_bits + GIE); // Enter LPM3 with interrupts enabled

### Lab3A_2: Real Time Clock with Basic Timer1

Project files

- C source files:	Chapter 7 > Lab3 > Lab3a_2_student.c

Chapter 7 > Lab3 > LCD_defs.h

- Solution file:	Chapter 7 > Lab3 > Lab3a_2_solution.c

Overview

The Real Time Clock (RTC) has a 32-bit counter, to automatically control the clock calendar. This peripheral is present on the  MSP430FG461x devices. The application developed in the previous laboratory will now be modified to incorporate this module.

A. Resources

This application is based on the same resources used in the previous laboratory. In addition, there is an additional RTC peripheral and two push buttons, SW1 and SW2. The first module works in automatic mode to manage the clock calendar, while the push buttons switch the information displayed on the LCD between the clock and calendar.

B. Software application organization

The organization of the software is identical to that of LAB3A_1 laboratory. The Basic Timer1, LCD and LEDs continue to perform the same functions. They are configured similarly, but with the changes described below.

In routine main(), the configurations for RTC and SW1/SW2 are added.

The memory addresses corresponding to the clock calendar values are initialized with the default values, that is zero hours, zero minutes and zero seconds, on August 9, 2008. The RTC is then activated in calendar mode, with the interrupt disabled. This mode affects the Basic Timer1 operation.

The switches SW1 and SW2 are connected to the microcontroller ports P1.0 and P1.1 respectively. Hence, these ports are configured as inputs and their interrupts activated by a high-to-low transition at the input.

C. System configuration

    - Real Time Clock configuration

The RTC is configured in calendar mode and enabled. The counting registers provide the values of seconds, minutes, hours, days, day of the week, day of the month, month and year. The registers are stored in BCD format to speed up the data writing process to the LCD. The interrupt for this peripheral should be disabled (disabling the Basic Timer1 interrupt). Given these objectives, what is the configuration value of the following register?

RTCCTL = ____________________;

The RTC operation in calendar mode automatically configures some of the Basic Timer1 features. The content of the bits BTSSEL, BTHOLD and BTDIV of BTCNT register are ignored. Thus, the BTCNT1 and BTCNT2 counters work in cascade. The clock source of the BTCNT1 counter is the ACLK clock signal. The output of the BTCNT1.Q7 counter is selected as the input of the BTCNT2 counter (frequency: ACLK/256). The RTC uses the BTCNT2.Q6 output as clock source (frequency: ACLK/32768).

    - Basic Timer1 configuration

This peripheral is automatically configured with the RTC in calendar mode. To enable the interrupt once every 0.5 seconds, what is the value to write to the following register:

BTCNT = ________________;

    - Ports P1.0 and P1.1 configuration

The switches SW1 and SW2 are connected to ports P1.0 and P1.1 respectively. How should the following registers be configured in order to set just the bits that affect the digital inputs, with high-to-low transition interrupts?

P1SEL &= _______________;

P1DIR &= _______________;

P1IFG  = _______________;

P1ES &= ________________;

P1IE |= ________________;

D. Analysis of the operation

Knowing what values and configurations to give each of the device’s registers, complete the blank spaces in the file LAB3a_2_student.c. The solution of the laboratory can be found in the file LAB3a_2_solution.c. One of these files should be included in the construction of the project. After compiling, debug the project.

    - ISR execution time

Performing similar procedures to those described in laboratory Lab3A_1 measure the ISR execution time. What is the value measured?

LCD refresh: ______

The LCD write routines were changed. Taking advantage of storing the data in the BCD format, the division operation can be ignored, resulting in the reduction of execution time of the Basic Timer1 ISR. Is the processing time required to refresh the LCD constant? _____

    - Measurement of electrical current drawn

Using the procedure similar to that described at the corresponding point of Lab3A_1, measure of the value of current drawn by the device.

What is the value measured? __________

| MSP-EXP430FG4618 | SOLUTION |
| --- | --- |
| Implement a Real Time Clock (RTC) using Basic Timer1. |  |

    - Real Time Clock configuration:

RTCCTL = RTCBCD | RTCHOLD | RTCMODE_3;        // BCD mode, RTC and BT disable

    - Basic Timer1 configuration:

BTCTL = BT_fCLK2_DIV64;            // (ACLK/256)/64

IE2 |= BTIE;        // Enable BT interrupt with 0.5 period

    - Ports P1.0 and P1.1 configuration:

P1SEL &= ~0x03;       // P1.0 and P1.1 as inputs

P1DIR &= ~0x03;       // P1.0 and P1.1 digital inputs

P1IFG  = 0x00;

P1IES &= ~0x03;       // high-to-low transition interrupts

P1IE |= 0x03;         // enable port interrupts

### Lab3B: Memory Clock with Timer_A

Project files

- C source files:	Chapter 7 > Lab3 > Lab3b_student.c

Chapter 7 > Lab3 > LCD_defs.h

- Solution file:	Chapter 7 > Lab3 > Lab3b_solution.c

Overview

The objective of this laboratory is to build a memory clock similar to the one that was developed using the Basic Timer1, in laboratory Lab3A_1. Timer_A is configured to generate an interrupt once every 100 msec. The ISR manages the memory clock. LED1 and LED2 are used to monitor the operation of the system state.

A. Resources

This application makes use of Timer_A to generate an interrupt when the value in the TACCR0 unit is reached. The ISR updates the contents of the memory clock variables.

LED1 monitors the system operation, switching state whenever the Timer_A ISR runs. LED2 can be used to monitor the ISR execution time. The contents of the LCD is updated every interrupt. When the ISR finishes, the device returns to low power mode.

Hence, the system resources used by this application are:

    - Timer_A;

    - I/O ports;

    - LCD;

    - Interrupts;

    - Low power modes.

The default configuration of the FLL+ is used, so all the clock signals required for the operation of the device assume their default values.

B. Software application organization

The first task is to disable the Watchdog Timer. All the resources needed for the LCD are then configured. The complete code is given, because its operation will be analysed in a later laboratory. Once configured, the LCD is cleared by the execution of the routine LCD_all_off().

The memory clock consists of three global variables: min, sec, msec, of the type unsigned char, to store the minutes, seconds and milliseconds respectively of the values elapsed since the beginning of the execution of the application. These variables are initialized with zeros.

The LCD is refreshed at startup to display the initial clock value.

LED2 is used as an indicator of Timer_A ISR execution. The execution time can be monitored using it. In addition, LED1 switches state whenever Timer_A ISR is executed.

Timer_A is configured to generate an interrupt once every 100 milliseconds.

The routine main() ends with a global interrupt enable and puts the device into a low power mode, where it waits for the next interrupt.

Timer_A ISR begins by activating LED2, indicating the beginning of execution of the routine and then switches LED1 state. The counters are updated in cascade and their contents are used to update the LCD, through the routines LCD_msec(), LCD_sec() and LCD_min(). The routine ends by switching the state of the clock separation points. Finally, LED2 is turned off.

C. System configuration

    - Disable the Watchdog Timer

The Watchdog Timer is configured as in the above examples.

    - FLL+ configuration

FLL+ is configured as in the above examples.

    - LED ports configuration

LED ports are configured as in the above examples.

    - Timer_A configuration

The Timer_A is configured to count until it reaches the value written in the TACCR0 unit. An interrupt is generated when it reaches that value. Which is the interrupt vector to use? ____________

Timer_A clock signal is the ACLK without division. What is the value to write in the configuration register?

TACTL = _____________;

The TACCR0 capture/compare unit determines the Timer_A counting range. For a 100 msec response, what is the value to write in the register?

TACCR0 = ____________;

The interrupt is configured in TACCR0 capture/compare unit. What is the value to write to the following register?

TACCTL0 = _____________;

    - Low power mode

The low power mode is configured as in the above examples.

D. Operation analysis

Knowing the values and configurations for each register, complete the file LAB3b_student.c. The complete solution can be found in the file LAB3b_solution.c. One of these files should be included in building the project. After compiling, debug the project.

    - ISR execution time

Using similar procedures to those described in laboratory Lab3A measure the ISR execution time. What is the value measured?

LCD refresh: ______

    - Measurement of electrical current drawn

Using a procedure similar to that described at the corresponding point of Lab3A_1, measure of the value of current drawn by the device.

What is the value measured? __________

| MSP-EXP430FG4618 | SOLUTION |
| --- | --- |
| Implement a Memory Clock using Timer_A. |  |

Timer_A configuration:

TACTL = TASSEL_1 | MC_1 | ID_0;   // ACLK, up mode

TACCR0 = 3268;       // this count corresponds to 100 msec

TACCTL0 = CCIE;      // TACCR0 interrupt enabled

### Lab3C: Buzzer tone generator

Project files

- C source files:	Chapter 7 > Lab3 > Lab3c_student.c

- Solution file:	Chapter 7 > Lab3 > Lab3c_solution.c

Overview

The purpose of this laboratory is to build a sound generator using Timer_B. The PWM signal produced by this peripheral drives the buzzer, producing a sequence of musical notes at regular time intervals. At the same time, LED1 and LED2 switch state alternately. The volume of sound produced by the buzzer can be controlled by push buttons SW1 and SW2.

A. Resources

The implementation of this application requires the production of specific frequency signals corresponding to musical notes. For each frequency, the duty-cycle can be modified in order to control the volume of sound produced. This task is carried out using Timer_B and one of its compare units. The buzzer is operated by Port P3.5, configured to work in its special function as TB4 compare unit output. This output corresponds to the TBCCR4 output compare unit.

The push buttons SW1 and SW2 are connected to ports P1.0 and P1.1 respectively. An interrupt is generated when either of these buttons are pressed. The duty-cycle of the generated note is modified in response.

Basic Timer1 is configured to generate an interrupt once every second. The interrupt service routine updates the musical notes produced by the buzzer, which are stored in an array.

LED1 and LED2 are driven from P2.2 and P2.1 respectively, and their state is switched alternately once every second.

The module FLL+ is configured to a 7.995392 MHz frequency, for the MCLK and SMCLK clock signals.

The resources used by the application are:

    - Timer_B;

    - Basic Timer1;

    - I/O ports;

    - FLL+;

    - Interrupts.

B. Software application organization

The application consists of the routine main(), which is used to configure all system resources, before entering into a standby mode, waiting for one of two interrupts.

This routine starts by disabling the watchdog timer and starting the module FLL+ to produce the desired clock signals of the correct frequency for the SMCLK and MCLK. Then, the Basic Timer1 and Timer_B are configured in order to perform the desired functions.

The ports connected to the LEDs, buttons and buzzer are then initialized.

Finally, the interrupts are activated, and the application waits for the execution of one of two interrupts.

The Basic Timer1 interrupt executes at a frequency of once every second. When this interrupt is occurs, it begins by switching the state of LED1 and LED2. Afterwards, it accesses the memory to fetch the next musical note to be performed. The routine ends with memory pointer management.

The Port 1 ISR begins by evaluating the source of the interrupt. The sound volume is reduced if the button SW1 is pressed. The sound volume is increased if button SW2 is pressed.

C. System configuration

    - Timer_B

It is the responsibility of Timer_B to produce the PWM signal that activates the Buzzer. Timer_B counts until the value contained in the TBCCR0 register is reached. It does not generate an interrupt, and must be sourced by SMCLK clock signal.

What is the value to write to this configuration register?

TBCTL = _____________;

Each PWM signal produced by Timer_B corresponds to a musical note. The relationship between the frequency and the musical note is given in Table 7-3.

Table 7-3. Relationship table between the frequency and the musical note.

| Note | SI0 | DO | RE | MI | FA | SOL | LA | SI | DO2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Freq [Hz] | 503 | 524 | 587 | 662 | 701 | 787 | 878 | 1004 | 1048 |

Timer_B has a frequency clock input equal to 7.995392 MHz.

What is the value to write in the TBCCR0 register in order to generate the desired frequency?

Table 7-4. TBCCR0 value of the musical notes.

| Note | SI0 | DO | RE | MI | FA | SOL | LA | SI | DO2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TBCCR0 | ____ | ____ | ____ | ____ | ____ | ____ | ____ | ____ | ____ |

The TBCCR4 compare unit is used to produce the PWM signal. The set/reset compare mode is used.

What is the value to write to the configuration register?

TBCCTL4 = _______________;

The volume control consists of varying the PWM signal duty-cycle. Initially its default value is 50%. What is the configuration value to write to the register?

TBCCR4 = ________________;

    - Basic Timer1

The Basic Timer1 generates an interrupt once every second. It uses two counters in series, where the BTCNT2 counter input uses the BTCNT1 counter output divided by 256. The BTCNT1 counter input is the ACLK clock signal with a frequency of 32.768 kHz.

If BTCNT2 counter selected output is divided by 128, what is the time period associated with the Basic Timer1 interrupt? _________

What are the values to write to the configuration registers?

BTCTL = __________________;

IE2 = ____________________;

    - I/O Ports

Three ports are used by this application. In port P1, bits P1.0 and P1.2 are used to activate the ISR whenever the buttons SW1 and SW2 are activated.

How should just the bits related to these ports be configured in order to have digital input functions, with high-to-low transition interrupts?

P1SEL &= _________________;

P1DIR &= _________________;

P1IFG = __________________;

P1IES &= _________________;

P1IE |= __________________;

LED1 and LED2 are connected to ports P2.2 and P2.1 respectively. How should just the bits related to these ports be configured in order to have digital output functions?

P2DIR = ___________________;

Configure the P2OUT register in order to initialize the application with the LED1 on and the LED2 off.

P2OUT = ____________________;

Configure the P3 register, with P3.5 connected to the buzzer. Remember to configure it as special function output (Timer_B compare output – TB4).

P3SEL = ____________________;

P3DIR = ____________________;

    - FLL+ configuration

This module uses the 32.768 kHz frequency crystal to produce a 7.995392 MHz frequency at the SMCLK and MCLK clock signals. What are the values to write to the configuration registers?

FLL_CTL0 |= ________________;

SCFI0 |= ___________________;

SCFQCTL = __________________;

D. Analysis of operation

    - System clocks inspection

The MCLK, SMCLK and ACLK system clocks are available at ports P1.1, P1.4 and P1.5 respectively. These ports are located on the SW2, RESET_CC and VREG_EN lines, which are available on the H2 Header pins 2, 5 and 6. All these resources are available because the Chipcon RF module is not installed and SW2 is not used.

Using the Registers view, set bits 1, 4 and 5 of P1SEL and P1DIR registers to choose the secondary function of their ports, that is, configured as outputs. Connect an oscilloscope probe at these positions to monitor the clock signals.

What are the values measured for each of the system clocks?

ACLK: _____________________

SMCLK: ____________________

MCLK: _____________________

    - TBCCR4 unit output frequency

With the help of an oscilloscope, it is possible to evaluate the operation of the application. Alternatively, it is possible to listen to the sound produced. By removing jumper JP1 and connecting the oscilloscope to this pin, it is possible to view the PWM signal produced by the microcontroller. The duty-cycle can be reduced or increased by pressing the push buttons SW1 and SW2.

    - Port P1 interrupt source decoding

All Port P1 interrupt lines share the same interrupt vector. The decoding is done through the P1IFG register.

This process can be observed by entering a breakpoint at the first line of the ISR code.

Execute the application.

The application’s execution is suspended at the breakpoint by pressing either button SW1 or SW2. From this point onwards, run the lines of code step-by-step and observe changes in the register values.

    - Measurement of electrical current drawn

Using a procedure similar to that described at the corresponding point of Lab3A_1, measure of the value of current drawn by the device.

What is the value read? __________

| MSP-EXP430FG4618 | SOLUTION |
| --- | --- |
| Implement a Buzzer tone generator. |  |

    - Timer_B configuration:

// SMCLK, continuous mode

TBCTL = TBSSEL_2 | CNTL_0 | TBCLGRP_0 |MC_1 | ID_0;

// TBCCR0 value of the musical notes

#define SI0 15895

#define DO 15258

#define RE 13620

#define MI 12077

#define FA 11405

#define SOL 10159

#define LA 9106

#define SI 7963

#define DO2 7629

TBCCTL4 = OUTMOD_3;              // CCR4 interrupt enabled

TBCCR4 = space[0]/2;

    - Basic Timer1 configuration:

BTCTL = BTDIV | BT_fCLK2_DIV128;    // (ACLK/256)/128

IE2 |= BTIE;                        // enable BT interrupt

    - I/O Ports configuration:

// SW1 and SW2 configuration (Port1)

P1SEL &= 0x00;         // P1.0 and P1.2 I/O

P1DIR &= 0x00;         // P1.0 and P1.2 as inputs

P1IFG = 0x00;

P1IES &= 0xFF          // high-to-low transition interrupt

P1IE |= 0xFF;          // enable port interrupts

// LED1 and LED2 configuration (Port2):

P2DIR |= 0x06;        // P2.2 and P2.1 as outputs

P2OUT = 0x04;         // LED1 on and LED2 off

// Buzzer port configuration (Port3)

P3SEL |= 0x20;        // P3.5 as special function

P3DIR |= 0x20;        // P3.5 as digital output

    - FLL+ configuration:

FLL_CTL0 |= DCOPLUS + XCAP18PF; //DCO+ set,freq=xtal*D*N+1

SCFI0 |= FN_4;            // x2 DCO freq, 8MHz nominal DCO

SCFQCTL = 121;           // (121+1) x 32768 x 2 = 7.99 MHz

### Lab3D: Frequency measurement

Project files

- C source files:	Chapter 7 > Lab3 > Labd_student.c

Chapter 7 > Lab3 > LCD_defs.h

- Solution file:	Chapter 7 > Lab3 > Labd_solution.c

Overview

This laboratory implements an application designed to measure a PWM signal frequency. If a signal generator is not available, the microcontroller generates a PWM signal based on the frequencies stored in a file. The frequencies generated are read and updated with a fixed time period using the features of CCE. The measured value is shown on the LCD in Hz.

A. Resources

The module FLL+ is configured to a frequency of 7.995392 MHz for the MCLK and SMCLK clock signals. This application performs the two tasks simultaneously.

On the one hand, it generates a PWM signal with a frequency of 200 Hz and a duty cycle of 50%. Alternatively, the PWM signal frequency can be read from a file using a breakpoint. This function is performed by Timer_B, using the compare unit to generate the PWM signal.

The time period between two consecutive PWM signals low-to-high transitions is measured by Timer_A. The capture unit of this timer is configured to collect the Timer_A counter register’s contents when a PWM signal low-to-high transition is detected at its input.

The Basic Timer1 generates an interrupt once every second. The ISR updates the PWM signal frequency generated by the Timer_B. If you choose to use this feature, a breakpoint associated with this ISR execution allows reading a file with the value of the frequency that will be generated.

The microcontroller’s ports are configured in order that the PWM signal generated by Timer_B through the TBCCR4 compare unit available at Port P3.5/TB4 can be connected to the Port P1.2/TA1 of the Timer_A TACCR1 capture unit. If you plan to use this feature, these pins must be connected together. Port P3.5 pin is available on Header 7 pin 6, while the Port P1.2 pin is available on Header H2 pin 3.

Ports P2.1 and P2.2 are used to monitor the state of the LED2 and LED1, respectively.

The resources used by the application are:

    - Timer_A;

    - Timer_B;

    - Basic Timer1;

    - I/O ports;

    - FLL+;

    - Interrupts.

B. Software application organization

The software structure allows various tasks to be performed simultaneously. The routine main() is responsible for configuring all the resources used by the application. Once started, the application enables all the interrupts and waits for an interrupt request.

There are two routines that separately service the two possible interrupts. The routine TimerA1_ISR() services interrupts required by the Timer_A overflow and by the TACCR1 capture unit. For every interrupt caused by a TACCR1 capture, the value collected in the  TACCR1 register is stored in T1, if it is the first low-to-high transition, or stored in T2 if it is the second low-to-high transition. This sequence is controlled by the variable capture. The variable flag is used to flag the measurement process. This process occurs between the capture of the first low-to-high transition and the second transition. The counting of clock pulses is done by Timer_A, in the time interval between the T1 and T2 acquisition, assigned to the variable T. The process is synchronized when Timer_A overflows, restarting the measurement process. The LCD is refreshed once every 0.5 seconds with the latest measured frequency value, using the control variable control tick that corresponds to 0.5 seconds.

The routine basic_timer_ISR() services the interrupt produced by  Basic Timer1 once every second. This routine begins by switching the state of LED1 and LED2. In addition, it updates the Timer_B counting period. The variable read_data allows the counting period to be changed.

C. System configuration

    - Basic Timer1

Basic Timer1 generates an interrupt once every second. Use the two counters in series, where the BTCNT2 counter input is selected as the BTCNT1 counter output divided by 256. The BTCNT1 counter input is the ACLK clock signal with a frequency of 32.768 kHz.

If BTCNT2 counter selected output is divided by 128, what is the time period associated with the Basic Timer1 interrupt? _________

What are the values to write to the configuration registers?

BTCTL = ________________;

IE2 = __________________;

    - Timer_B

The TBCCR4 compare unit is used to generate the PWM signal. The set/reset compare mode is used.

What is the value to write to the configuration register?

TBCCTL4 = ______________;

The TB4 PWM output signal has a frequency X, with a 50% duty-cycle. The SMCLK clock signal is used as input of Timer_B.

What are the values to write to the registers?

TBCCR0 = _______________;

TBCCR4 = _______________;

What the largest and lowest generated frequency?

Maximum frequency value: ____________

Minimum frequency value: _____________

    - Timer_A

Timer_A is sourced by the SMCLK clock signal. It counts to the value 0xFFFF, in continuous mode. An interrupt is generated when the TAR counter overflows. What is the value to write to its configuration register?

TACTL = _________________;

The capture unit captures the TAR register value to the TACCR1 register when it detects a low-to-high transition at the TA1 input. What is the value to write to the configuration register?

TACCTL1 = _______________;

Determine the maximum and minimum frequency values detected. Note that these values do not take into account the execution time of the application. The PWM signals should be applied at frequencies well below the maximum value determined.

Maximum frequency value: ____________

Minimum frequency value: _____________

The TACCR1 capture unit is configured to generate an interrupt when it detects a low-to-high transition. What is the value to write to the configuration register?

TACCTL1 = _____________;

    - Ports P3.5/TB4 and P1.2/TA1 configuration

These ports perform special functions. Thus, the Port P3.5 is configured as an output, selected by the special function TB4, with the values:

P3SEL = _______________;

P3DIR = _______________;

The Port P1.2 is configured as input, with the special function TA1, using the values:

P1SEL = _______________;

P1DIR = _______________;

    - LED ports configuration

The LEDs should be configured as in Lab3C.

D. Analysis of operation

    - Run the application using the frequency generator based on Timer_B

Without a frequency generator, the Timer_B generates a PWM signal at the TBCCR4 unit output that can be fed back to Timer_A TACCR1 capture unit input. These two pins must therefore be connected together. By default, the PWM signal frequency is 200 Hz. Add a breakpoint at the line of code 233 belonging to the Basic Timer1 ISR to modify this value.

TBCCR0 = 7995392/read_data;

If the variable read_data has the value 200, it will generate a 200 Hz frequency. The value of this variable can be changed by associating a breakpoint to that line of code. Before the line of code is executed, the value of the data file is read and assigned to the variable read_data. The signal will oscillate at the desired frequency, loading the value in TBCCR0. The breakpoint configuration is as follows:

    - Action: read data from file

    - File: address of the data file (example in freq.txt)

    - Wrap Around: activate this option to restart reading at the beginning

    - Start address: &read_data

    - Length: 1 in order to read a value from the file each time

Alternatively, the breakpoint can be imported from the file Lab3d_breakpoint.bkpt to debug. The file has a format already described in the chapter devoted to the CCE. Each line describes in Hz the value of the PWM signal frequency.

    - Run the application using a frequency generator

The operation of the application can be verified using a frequency generator. The generator should generate a PWM signal with voltage and frequency values compatible with the device’s input (2.5 to 3.3 volts).

    - Observe the measured frequency

The PWM signal applied to the TA1 input can be viewed using an oscilloscope, connected to pin 3 of Header 2. Perform this task and confirm the values present at the LCD.

    - Measurement of electrical current drawn

Carry out the procedure as described in previous laboratories for measuring the device current.

What is the value read? __________

| MSP-EXP430FG4618 | SOLUTION |
| --- | --- |
| Perform a PWM signal frequency measurement. |  |

    - Basic Timer1 configuration:

BTCTL = BTDIV | BT_fCLK2_DIV128;         // (ACLK/256)/128

IE2 |= BTIE;      // Enable BT interrupt with 1 sec period

    - Timer_B configuration:

TBCTL = TBSSEL_2 | CNTL_0 | TBCLGRP_0 |MC_1 | ID_0;

// SMCLK, continuous mode

TBCCTL4 = OUTMOD_3;      // CCR4 output mode 3 (set/reset)

TBCCR0 = 39977;// Output 200 Hz signal with 50% duty cycle

TBCCR4 = TBCCR0/2;

    - Timer_A configuration:

TACTL = TASSEL_2 |MC_2 | ID_0 | TAIE;

// SMCLK, up mode to 0xFFFF

TACCTL1 = CM1 | CCIS_0 | CAP | CCIE;

// Capture on rising edge, TACCR1 input signal selected, Capture mode, Capture/compare interrupt enable.

    - Ports P3.5/TB4 and P1.2/TA1 configuration:

// TB4 configuration (Port3)

P3SEL = 0x20;           // P3.5 as special function (TB4)

P3DIR = 0x20;           // P3.5 as output

// TA1 (TACCR1) configuration (Port1)

P1SEL = 0x04;           // P1.2 as special function (TA1)

P1DIR = 0x00;           // P1.2 as input

## Quiz

1. The timer/counter suitable for LCD controller frame frequency generation in the MSP430FG4618 is:

(a) Timer_A;

(b) BTCNT2;

(c) Timer_B;

(d) BTCNT1.

2. To set Timer_A to repeatedly count from 0x000 to 0xFFFF, the operating mode selected must be:

(a) Up/Down mode (MCx = 3);

(b) Up mode (MCx = 1);

(c) Continuous (MCx = 2);

(d) Stop (MCx = 0).

3. Timer_A configured in continuous operating mode is reset with:

(a) Write 0xFFFF to TAR register;

(b) Reset TACCR0;

(c) Set TACLR bit in the TACTL register;

(d) None of the above.

4. When TASSELx = 1, the timer (Timer_A or Timer_B) is sourced by:

(a) ACLK;

(b) TACLK;

(c) INCLK;

(d) SMCLK.

5. To generate a PWM signal with active high state, it is necessary to configure the output operating mode to:

(a) Mode 2;

(b) Mode 3;

(c) Mode 6;

(d) Mode 7.

6. With Timer_A sourced by the ACLK (32768 Hz) and is configured in compare mode, the value in TACCR0 register to enable interrupts once each second is:

(a) 32768;

(b) 32767;

(c) 16384;

(d) 65536;

## FAQs

1. What are the Basic Timer1 registers initial condition?

None. They must be configured by user software before use.

2. What is the result of reading or writing to BTCNT1 and to BTCNT2 when CPU clock and counter clock are asynchronous:

Any read may be unpredictable. Any write take effect immediately.

3. Can Timer_A registers be modified during its operation?

Yes, if they are the TAIE, TAIFG or TACLR. For other configuration types, it is recommended to stop the timer before modifying its operation to avoid erroneous operating conditions.

4. Can the output mode be switched during the operation of a timer?

It is recommended to the remaining bits of OUTMODx bits to 1s during the transition between output modes, or otherwise to configure mode 7 as transition state while switching between output modes.